SECTION 101100 - Visual Display Surfaces

- GENERAL

      - SUMMARY

        - Related Documents:

          - Drawings and general provisions of the Subcontract apply to this Section.

          - Review these documents for coordination with additional requirements and information that apply to work under this Section.

        - Section Includes:

          - Prefabricated markerboard and prefabricated tackboard.

        - Related Sections:

          - Division 01 Section "General Requirements."

          - Division 01 Section "Special Procedures."

<Select among the following as appropriate.>

          - Division 05 Section "Cold-Formed Metal Framing".

          - Division 06 Section "Architectural Woodwork" for wood framed tackboards.

          - Division 09 Section "Non-Structural Metal Framing".

          - Division 09 Section "Gypsum Board."

      - SUBMITTALS

        - Submit under provisions of Division 01 Section "General Requirements."

        - Product Data: Submit manufacturer's specifications and instructions for installation, for markerboards, tackboards, and related accessories. University's review will be for color and style only. Compliance with all other requirements is the exclusive responsibility of the Subcontractor.

        - LEED Submittals:

<Retain first subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.>

          - Product Data for Credit EQ 4.4:  For composite wood products, documentation indicating that the product contains no urea formaldehyde.

<Retain subparagraph below for visual display wall coverings and directly applied tack assemblies, if applicable, and if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1.>

          - Product Data for Credit EQ 4.1:  For adhesives, including printed statement of VOC content and chemical components.

- PRODUCTS

      - MATERIALS

        - Marker Board: Nominal 9/16 inch  thick specifically designed for use with liquid chalk markers, composed of 1/2 inch (3 mm) thick wood or hardboard core and porcelain enamel finished 24 gauge steel securely bonded to each other under pressure, using a waterproof adhesive. The exposed metal surface face shall comply with the requirements for markerboard surfaces as recommended by the Porcelain Enamel Institute. Color of markerboard shall be [color no. and name][ as scheduled on Drawings].

        - Tackboard: Cork composition with waterproof binder, flame retardant treated so as to provide a Class B rating with Flame Spread not to exceed 60, Fuel Contributed not to exceed 5 and Smoke Developed not to exceed 25, nominal 1/4 inch (6 mm) thick, with burlap jute backing. Cork material shall be asbestos free. Color shall be [color no. and name][as scheduled on the Drawings].

        - Adhesives for Mounting: As recommended by the manufacturer of the markerboard or tackboard.

        - Metal Trim: Extruded aluminum shapes as is standard with the manufacturer of the markerboard or tackboard, 204 clear anodized. Trim shall be "snap-on" type, or shall have concealed fasteners. Include vertical edge moldings, continuous chalk trough, and display/map rail as specified below:

          - Display/map rail shall be an extruded aluminum shape, with a continuous cork strip, approximately 2 inches wide, and an integral map rail with the following items:

            - End stops; removable type - one at each end of map rail.

            - Map hooks with flexible metal clips; two for each 4-foot section of map rail, or fraction thereof.

          - Where joints occur in markerboard or tackboard, provide extruded aluminum divider strips; finish to match other trim.

- EXECUTION

      - PREPARATION

        - Examine all surfaces to which markerboard and/or tackboards will be applied and notify the Project Subcontractor in writing of all conditions detrimental to the timely completion of the work. Do not proceed with the work of this Section until all unsatisfactory conditions have been corrected in a manner acceptable to the manufacturer of the materials.

      - INSTALLATION

        - Markerboard: Markerboard units shall be mounted by means of mechanical fasteners, size and spacing as recommended by the manufacturer for the type of installation involved. Units shall be of the size, and mounted at the height shown on the drawings. Units less than 12 feet (3.6m) in length shall have no vertical joints. Units less than 4-foot (1.2m) in height shall have no horizontal joints.

        - Tackboard: Store the material in the room or space in which it will be installed, for a minimum of 24 hours prior to the time of installation. Units shall be mounted by means of mechanical fasteners, size and spacing as recommended by the manufacturer of the tackboard for the type of installation involved. Units shall be of the size, and mounted at the height shown on the drawings. Units less than 12 feet (3.6m) in length shall have no vertical joints. Units less than 4-foot in height shall have no horizontal joints. Where joints do occur, they shall be flush, hairline, pattern matched where applicable, and in locations accepted by the Project Manager.

        - Metal Trim: Install in single continuous pieces for lengths up to 16 feet (4.8m). All corners shall be mitered, and all joints shall be butted to produce flush hairline joints, free of sharp cut edges or rough spots. Secure trim with concealed fasteners.

      - CLEANING

        - After completion of installation of the markerboard and tackboard materials, clean all surfaces with a cleaner or detergent as recommended by the manufacturer of the units. Replace all materials damaged during installation.

END OF SECTION 101100