SECTION 321540 - Crushed Stone Surfacing

- GENERAL

      - SUMMARY

        - Related Documents:

          - Drawings and general provisions of the Subcontract apply to this Section.

          - Review these documents for coordination with additional requirements and information that apply to work under this Section.

        - Section Includes:

          - Crushed stone paving course, compacted.

        - Related Sections:

          - Division 01 Section "General Requirements."

          - Division 01 Section "Special Procedures."

          - Division 31 Section "Rough Grading: for preparation of site for paving.

          - Division 31 Section "Backfilling" for compacted fill for paving.

      - REFERENCES

        - General:

          - The following documents form part of the Specifications to the extent stated. Where differences exist between codes and standards, the one affording the greatest protection shall apply.

          - Unless otherwise noted, the referenced standard edition is the current one at the time of commencement of the Work.

          - Refer to Division 01 Section "General Requirements" for the list of applicable regulatory requirements.

        - American Association of State Highway and Transportation Officials:

          - AASHTO T 27	Sieve Analysis of Fine and Coarse Aggregates.

- PRODUCTS

      - MATERIALS

        - Coarse Stone:  [Angular,] [Crushed,] [            ,] [washed] natural stone; free of shale, clay, friable materials and debris; graded [in accordance with AASHTO T 27] [within the following limits:

| Sieve Size | Percent Passing |
| --- | --- |
| 2 inches (50 mm) | [100] [ ] |
| 1 inch (25 mm) | [95] [ ] |
| 3/4-inch (20 mm) | [95] [ ] to [100] [ ] |
| 5/8 inch (16 mm) | [75] [ ] to [100] [ ] |
| 3/8-inch (10 mm) | [55] [ ] to [85] [ ] |
| No. [4] [ ] | [35] [ ] to [60] [ ] |
| No. [16] [ ] | [15] [ ] to [35] [ ] |
| No. [40] [ ] | [10] [ ] to [25] [ ] |
| No. [200] [ ] | [5 ] [ ] to [10] [ ]] |

        - Sand:  Natural river or bank sand; [washed,] free of silt, clay, loam, friable or soluble materials, and organic matter.

- EXECUTION

      - INSPECTION

        - A.	The University may retain the services of an independent inspection agency to verify that compacted [subgrade] [granular base] [stabilized soil] [        ] is dry and ready to receive work of this section.

        - The University may retain the services of an independent surveyor to verify that gradients and elevations of base are correct.

        - Beginning of installation means acceptance of existing conditions.

      - PLACING STONE PAVING

        - Spread stone material over prepared base to a total compacted thickness of <Insert Number Here> inches.

        - Place stone in 3 inches (75 mm) <Insert Number Here> layers and compact.

        - Level surfaces to elevations and gradients indicated.

        - Add small quantities of sand to stone mix as appropriate to assist compaction.

        - Compact placed stone materials to achieve <Insert Number Here> lb/cu. ft (kg/cu. M) dry density.

        - Add water to assist compaction.  With an excess water condition, rework topping and aerate to reduce moisture content.

        - Perform hand tamping in areas inaccessible to compaction equipment.

END OF SECTION 321540